CO3 – AT3 ERROR ANALYSIS TASK

Course Name: DIGITAL SIGNAL PROCESSING

Course Code: ECA0903

Student Name: PAMURU SANDEEP

Register Number: 192412632

Question 25

A 12-bit uniform quantizer with an input range of ±2 V gives a quantization step size of 0.002 V. Verify the calculation, identify the error, and compute the correct step size and maximum quantization error. (10) (BL4) (WK4)

1. Given Data

Number of bits, N = 12

Input range = ±2 V

Total input range = 4 V

Reported quantization step size = 0.002 V

2. Quantization Step Size Formula

For a uniform quantizer, the quantization step size is calculated using:

Δ = Total Input Range / Number of Quantization Levels

The number of quantization levels for an N-bit quantizer is:

L = 2^N

3. Calculation of the Correct Step Size

For a 12-bit quantizer:

L = 2^12 = 4096 levels

The total input range is from −2 V to +2 V:

Total Range = 2 − (−2) = 4 V

Therefore:

Δ = 4 / 4096

Δ = 0.0009765625 V

Thus, the correct quantization step size is approximately:

Δ ≈ 0.000977 V

4. Identification of the Error

The reported step size of 0.002 V is incorrect. It is approximately twice the correct value. The likely calculation error is using the wrong number of quantization levels or incorrectly using the input range in the formula. A 12-bit quantizer provides 4096 quantization levels, so the full 4 V input range must be divided by 4096.

5. Maximum Quantization Error

For an ideal uniform quantizer using rounding, the maximum quantization error is equal to half of one quantization step:

Maximum Quantization Error = Δ / 2

Maximum Quantization Error = 0.0009765625 / 2

Maximum Quantization Error = 0.00048828125 V

Therefore, the maximum quantization error is approximately:

±0.000488 V

6. Corrected Solution

Correct number of quantization levels = 4096

Correct step size = 4 / 4096 = 0.0009765625 V

Maximum quantization error = Δ / 2 = ±0.00048828125 V

Hence, the reported value of 0.002 V is incorrect.

7. DSP Justification

Increasing the number of quantization bits increases the number of available quantization levels and reduces the quantization step size. A smaller step size reduces the difference between the original and quantized values, thereby reducing quantization error and quantization noise. For a uniform quantizer, the maximum rounding error is limited to half of one quantization step.

8. MATLAB Code

clc;
clear;
close all;
N=12;
Vmin=-2;
Vmax=2;
levels=2^N;
step=(Vmax-Vmin)/levels;
max_error=step/2;
fprintf('Number of Quantization Levels = %d\n',levels);
fprintf('Correct Quantization Step Size = %.9f V\n',step);
fprintf('Maximum Quantization Error = ±%.9f V\n',max_error);

9. Expected MATLAB Output

Number of Quantization Levels = 4096
Correct Quantization Step Size = 0.000976562 V
Maximum Quantization Error = ±0.000488281 V

10. Conclusion

The reported quantization step size of 0.002 V is incorrect for a 12-bit uniform quantizer with an input range of ±2 V. The total range is 4 V and the quantizer has 4096 levels. Therefore, the correct step size is approximately 0.000977 V. The maximum quantization error is half of the step size, which is approximately ±0.000488 V.